1. Integrated raw data

All NaN values are structural — they arise because a field is not applicable to a given vehicle type, not due to data collection failure.

| Column Name | Type | NaN count & cause | Description |
| --- | --- | --- | --- |
| Model_Year | Float | NaN = 7,385 (Conventional/Hybrid) | Model year (2012–2026). NaN for all Conventional/Hybrid — the NRCan CO₂ Canada source does not include a model year column. |
| Make | String | None | Manufacturer name (e.g., ACURA, FORD, TESLA). |
| Model | String | None | Vehicle model name. May include trim suffixes: 4WD, AWD, FFV, SWB, LWB, EWB. |
| Vehicle_Class | String | None | Body/size class (e.g., Compact, Mid-Size, SUV - Small, Full-Size Pickup Truck). |
| Vehicle_Type | String | None | Drivetrain category: Conventional/Hybrid ¦ Battery Electric (BEV) ¦ Plug-in Hybrid (PHEV). |
| Engine_Size_L | Float | NaN = 1,195 (BEV) | ICE displacement in litres (0.6–8.4 L). NaN for BEVs which have no combustion engine. |
| Cylinders | Float | NaN = 1,195 (BEV) | Number of engine cylinders (2–16). NaN for BEVs. |
| Motor_kW | Float | NaN = 7,385 (Conventional/Hybrid) | Peak electric motor output in kW (35–930). NaN for Conventional/Hybrid which have no electric motor. |
| Transmission | String | None | Full transmission code including gear count (e.g., AS6, M5, AV, A1). See code table in Section 1.2. |
| Fuel_Type_Primary | String | None | Primary fuel: X = regular gasoline, Z = premium gasoline, D = diesel, E = ethanol (E85), N = natural gas, B = electricity. |
| Fuel_Type_Secondary | String | NaN = 8,580 (Conventional/Hybrid + BEV) | Secondary fuel for PHEV charge-sustaining mode (X or Z). NaN for all non-PHEV rows. |
| Fuel_Cons_City_L100km | Float | NaN = 1,195 (BEV) | City fuel consumption in L/100 km (4.2–30.6). NaN for BEVs. |
| Fuel_Cons_Hwy_L100km | Float | NaN = 1,195 (BEV) | Highway fuel consumption in L/100 km (4.0–20.6). NaN for BEVs. |
| Fuel_Cons_Comb_L100km | Float | NaN = 1,195 (BEV) | Combined fuel consumption (55% city / 45% hwy) in L/100 km (4.1–26.1). NaN for BEVs. |
| Fuel_Cons_Comb_mpg | Float | NaN = 1,589 (BEV + PHEV) | Combined fuel consumption in miles per imperial gallon (11–69). Only available for Conventional/Hybrid records. |
| Elec_Cons_City_kWh100km | Float | NaN = 7,779 (Conventional/Hybrid + PHEV) | City electricity consumption in kWh/100 km (13.7–45.2). Only available for BEVs. |
| Elec_Cons_Hwy_kWh100km | Float | NaN = 7,779 (Conventional/Hybrid + PHEV) | Highway electricity consumption in kWh/100 km (14.8–49.8). Only available for BEVs. |
| Elec_Cons_Comb_kWh100km | Float | NaN = 7,385 (Conventional/Hybrid) | Combined electricity consumption in kWh/100 km (1.8–47.3). NaN for Conventional/Hybrid. |
| Elec_Range_km | Float | NaN = 7,385 (Conventional/Hybrid) | All-electric range on a full charge in km (6–837). NaN for Conventional/Hybrid. |
| Total_Range_km | Float | NaN = 7,385 (Conventional/Hybrid) | Total driving range in km. BEVs = Elec_Range_km. PHEVs = electric + gasoline combined (92–1,035 km). NaN for Conventional/Hybrid. |
| Recharge_Time_h | Float | NaN = 7,385 (Conventional/Hybrid) | Estimated Level-2 recharge time in hours (1.1–18.6). NaN for Conventional/Hybrid. |
| CO2_Emissions_g_km | Integer | None (BEV = true 0) | ★ TARGET VARIABLE. Tailpipe CO₂ in g/km for combined driving (0–522). BEVs = 0 (no tailpipe emissions). |
| CO2_Rating | Float | NaN = 7,439 | Government CO₂ rating on a 1–10 scale (higher = cleaner). Only available for BEV/PHEV in recent model years. |
| Smog_Rating | Float | NaN = 7,482 | Government smog emissions rating 1–10 (higher = cleaner). Only available for BEV/PHEV in recent model years. |

Code Reference

| Code | Transmission type |
| --- | --- |
| A | Automatic |
| AM | Automated manual |
| AS | Automatic with select shift |
| AV | Continuously variable (CVT) |
| M | Manual |
| 3–10 | Number of gears (appended to type code) |

| Code | Fuel type |
| --- | --- |
| X | Regular gasoline |
| Z | Premium gasoline |
| D | Diesel |
| E | Ethanol (E85) |
| N | Natural gas |
| B | Electricity (BEV & PHEV primary) |

- Data for clustering

NaN values have been replaced with 0 (numeric) or "Unknown" (string) for clustering compatibility.

★ represents new features.

| Column Name | Type | NaN → value (count) | Description |
| --- | --- | --- | --- |
| vehicle_id ★ | String | None | Unique row key: "Make_Model_VehicleClass_VehicleType_Transmission". Acts as index for clustering; not a model feature. |
| Make | String | None | Manufacturer name. Identical to Dataset 1. |
| Model | String | None | Vehicle model name. Identical to Dataset 1. |
| Vehicle_Class | String | None | Body/size class. Identical to Dataset 1. |
| Vehicle_Type | String | None | Drivetrain category: Conventional/Hybrid ¦ BEV ¦ PHEV. Identical to Dataset 1. |
| Fuel_Type_Primary | String | None | Primary fuel code (X/Z/D/E/N/B). Identical to Dataset 1. |
| Fuel_Type_Secondary | String | NaN → "Unknown" (7,476 rows) | Secondary fuel (X or Z for PHEVs). NaN replaced with "Unknown" for all non-PHEV rows. |
| Transmission | String | None | Full transmission code (e.g., AS6, M5, AV). Identical to Dataset 1. |
| Transmission_Type ★ | String | None | Transmission type extracted from Transmission field: A = automatic, AM = automated manual, AS = auto select-shift, AV = CVT, M = manual. |
| Num_Gears ★ | Float | NaN → 0 (320 rows) | Number of gears extracted from Transmission code (1–10). 0 for single-speed BEVs (A1) and CVTs with no stated gear count. |
| Is_SUV ★ | Integer | 0 = not SUV | Binary flag: 1 if Vehicle_Class contains 'SUV', else 0. Engineered for body-type segmentation. |
| Is_Truck ★ | Integer | 0 = not truck | Binary flag: 1 if Vehicle_Class contains 'Truck' or 'Pickup', else 0. Engineered for body-type segmentation. |
| Model_Year | Float | NaN → 0 (6,281 rows) | Model year (2012–2026 for non-zero). 0 for all Conventional/Hybrid (NaN imputed). |
| Engine_Size_L | Float | NaN → 0 (1,195 rows) | ICE displacement in litres (0.6–8.4 for non-zero). 0 for BEVs (NaN imputed). |
| Cylinders | Float | NaN → 0 (1,195 rows) | Number of engine cylinders (2–16 for non-zero). 0 for BEVs (NaN imputed). |
| Motor_kW | Float | NaN → 0 (6,281 rows) | Peak electric motor output in kW (35–930 for non-zero). 0 for Conventional/Hybrid (NaN imputed). |
| Fuel_Cons_City_L100km | Float | NaN → 0 (1,195 rows) | City fuel consumption in L/100 km (4.2–30.6 for non-zero). 0 for BEVs (NaN imputed). |
| Fuel_Cons_Hwy_L100km | Float | NaN → 0 (1,195 rows) | Highway fuel consumption in L/100 km (4.0–20.6 for non-zero). 0 for BEVs (NaN imputed). |
| Fuel_Cons_Comb_L100km | Float | NaN → 0 (1,195 rows) | Combined fuel consumption in L/100 km (4.1–26.1 for non-zero). 0 for BEVs (NaN imputed). |
| Elec_Cons_Comb_kWh100km | Float | NaN → 0 (6,281 rows) | Combined electricity consumption in kWh/100 km (1.8–47.3 for non-zero). 0 for Conventional/Hybrid (NaN imputed). |
| Elec_Range_km | Float | NaN → 0 (6,281 rows) | All-electric range in km (6–837 for non-zero). 0 for Conventional/Hybrid (NaN imputed). |
| Total_Range_km | Float | NaN → 0 (6,281 rows) | Total driving range in km (92–1,035 for non-zero). 0 for Conventional/Hybrid (NaN imputed). |
| Recharge_Time_h | Float | NaN → 0 (6,281 rows) | Level-2 recharge time in hours (1.1–18.6 for non-zero). 0 for Conventional/Hybrid (NaN imputed). |
| CO2_Emissions_g_km | Integer | None (BEV = true 0) | TARGET VARIABLE. Tailpipe CO₂ in g/km (14–522 for non-zero). BEVs = 0 is a true measurement, not imputed. |
| Fuel_Efficiency_Index ★ | Float | NaN → 0 (1,195 rows) | Derived: 100 ÷ Fuel_Cons_Comb_L100km. Higher = more fuel-efficient (3.83–24.39 for non-zero). 0 for BEVs. |
| CO2_per_L_engine ★ | Float | NaN → 0 (1,195 rows) | Derived: CO2_Emissions_g_km ÷ Engine_Size_L. CO₂ intensity per litre of displacement (15.5–197.0). 0 for BEVs. |
| City_Hwy_Fuel_Ratio ★ | Float | NaN → 0 (1,195 rows) | Derived: Fuel_Cons_City ÷ Fuel_Cons_Hwy. Values > 1 = higher city penalty (0.86–1.82). 0 for BEVs. |
| Electric_Efficiency_Index ★ | Float | NaN → 0 (6,281 rows) | Derived: 100 ÷ Elec_Cons_Comb_kWh100km. Higher = more energy-efficient (2.11–55.56 for non-zero). 0 for Conventional/Hybrid. |
| Range_per_ChargeHour ★ | Float | NaN → 0 (6,281 rows) | Derived: Elec_Range_km ÷ Recharge_Time_h. km of electric range per hour of charging (3.0–86.44). 0 for Conventional/Hybrid. |
| Elec_Range_Share ★ | Float | NaN → 0 (6,281 rows) | Derived: Elec_Range_km ÷ Total_Range_km. Proportion of total range from electric power (0.01–1.0; BEV = 1.0). 0 for Conventional/Hybrid. |